# CH-W-S-A22-P-E 2863100000

Mode 3 AC EV charging box SMART ECO with a maximum charging capacity of 22 kW at 3-phase (400 V) grid conection, maximum charging current of 32 A (Current limit can be set individually), with permanently attached charging cable (5m) and type 2 plug, integrated 6 mA residual current detection (DC), LEDs for status indication, robust IP54-housing, installation friendly housing concept for easy and fast assembly, different digital and serial Interfaces, networkable, updatable, mobile app for remote control and configuration

Technical Data

| Dimensions and weight: Dimensions: 268 x 433 x 150 mm Net weight: 6.1 kg Temperatures and storage: Storage temperature: -40 °C … 70 °C Operating Temperature: -30 °C … 45 °C Humidity: 5 … 95% rel. humidity Performance specification: Charge mode: Mode 3 according to IEC 61851-1 Input voltage type: AC Grid connection: 400 V AC 3-phase Max. charging capactiy: 22 kW Max. Charging current: 32 A (Adjustable in 1 A Steps) Min. Charging current: 6 A System requirements: Connection system: With attached 5 m cable and type 2 plug Type of communication to vehicle: Control Pilot (CP), Proximity Pilot / Plug Present (PP) Protection class: I Surge voltage category: III Pollution severity: 3 Features: Type of status indicator: RGB LED Welding detection: Yes Failure shutdown: Yes Failure Shutdown method: All-pole Integrated DC residual current protection: 6 mA Type of residual current detection to be provided by the Installation: RCD type A Mobile App available: Yes User authentification type: Mobile App Supported protocols: Modbus RTU, Modbus TCP | Interfaces: WiFi: Yes (2.4 GHz) Bluetooth module: Bluetooth low energy Ethernet ports: 1 Digital inputs: 5 (12 V / 6 mA) Digital outputs: 1 (12 V / 100 mA) Serial interfaces: RS485 Assembly: Assembly: Wall mounting, stand mounting Included in delivery: Included in delivery: Drilling template, Instruction sheet, Mounting screws, Assembly dowels Housing: Operating altitude: ≤ 3000 m Housing main material: PC Protection degree: IP54 Colour: black, Anthracite, grey Approvals and Norms: IEC 61851-1, IEC 61851-21-2, IEC 62196-1, IEC 62196-2, EN 50620, EN 301 489-1, EN 301 908-1, EN 301 511, EN300 328, EN 300 330 v.2.1.1, EN 62368-1, EN 55011 Connection data (supply cable): Conductor cross-section, max.: 16 mm² Manfacturer: Weidmüller |
| --- | --- |