# CH-W-S-A22-S-V 2875230000

Mode 3 AC EV charging box SMART VALUE with a maximum charging capacity of 22 kW at 3-phase (400 V) grid connection, charging switchable from 3- to 1-phase charging, maximum charging current 32 A (Current limit can be set individually), type 2 shutter socket for the connection of AC-charging cables, integrated 6 mA residual current detection (DC), LEDs for status indication, RFID/NFC reader for user authentification, integrated energy meter, robust IP54-housing, Installation friendly housing concept for an easy and fast installation, different digital and serial interfaces, remote controllable (e.c. via OCPP 1.6 with a backend), networkable ( including switch functionality with 2 ethernet ports), updatable, Mobile app for remote control and configuration, static load management

Technical Data

| Dimensions and weight: Dimensions: 273 x 439 x 167 mm Net weight: 4.3 kg Temperatures and storage: Storage temperature: -40 °C … 70 °C Operating Temperature: -30 °C … 45 °C Humidity: 5 … 95% rel. humidity Performance specification: Charge mode: Mode 3 according to IEC 61851-1 Input voltage type: AC Grid connection: 400 V AC 3-phasig Max. charging capactiy: 22 kW Max. Charging current: 32 A (Adjustable in 1 A Steps) Min. Charging current: 6A System requirements: Connection system: Type 2 socket outlet Type of communication to vehicle: Control Pilot (CP), Proximity Pilot / Plug Present (PP) Protection class: I Surge voltage category: III Pollution severity: 3 Features: Type of status indicator: RGB LED Welding detection: Yes Failure shutdown: Yes Failure Shutdown method: All-pole Integrated DC residual current protection: 6 mA Type of residual current detection to be provided by the installation: RCD Type A Mobile App available: Yes User identification type: Mobile App, RFID / NFC Energy measurement integrated: Yes Integrated load management: static Supported protocols: Modbus RTU, Modbus TCP, OCPP 1.6 (J) | Interfaces: WiFi: Yes (2.4 GHz) Bluetooth module: Bluetooth low energy Ethernet ports: 2 Digital inputs: 5 (12 V / 6 mA) Digital outputs: 1 (12 V / 100 mA) Serial interfaces: RS485 Assembly: Assembly: Wall mounting, stand mounting Included in delivery: Included in delivery: B Drilling template, Instruction sheet, 5 RFID-transponder, Mounting screws, Assembly dowels, cover Housing: Operating altitude: ≤ 3000 m Housing main material: PC Protection degree: IP54 Colour: black, Anthracite, grey Approvals and Norms: IEC 61851-1, IEC 61851-21-2, IEC 62196-1, IEC 62196-2, EN 50620, EN 301 489-1, EN 301 908-1, EN 301 511, EN300 328, EN 300 330 v.2.1.1, EN 62368-1, EN 55011 Connection data (supply cable): Conductor cross-section, max.: 16 mm² Manfacturer: Weidmüller |
| --- | --- |